TEXTILE FINISHING TECHNOLOGY

UNIT - 1

Introduction to Textile Finishes Finishing - Introduction to Textile finishes –Definition , Objectives of finishing, Importance of finishing and classification, merits and demerits of finishes.

INTRODUCTION:

In order to impart the required functional properties to the fiber or fabric, it is customary to subject the material to different types of physical and chemical treatments. For example, wash and wear finish for a cotton fabric is necessary to make it crease-free or wrinkle-free. In a similar way, mercerising, singeing, flame retardant, water repellent, waterproof, anti-static and peach finishing achieve various fabric properties desired by consumers.

The use of 100% synthetic textiles has increased considerably since the development of textured yarns made of filaments and the growing production of knit goods. The use of open weave has enabled the production of lighter, breathable, fabrics to ensure better wearing comfort.

The properties of petroleum-based synthetic fibers, most important among them being polyamide, polyester and polyacrylonitrile, are essentially different from those of natural cellulosic and protein/ wool fibers. Hence the sequence of finishing operations is likely to be different. While cellulosic fabrics require a resin finishing treatment to impart easy-care properties, synthetic fibers already exhibit these easy-care criteria and require only a heat setting operation.

DEFINITION:

A finish is any process that is done to fibre, yarn, or fabric either before or after fabrication to change the appearence ( what is seen), the hand ( what is felt), or the performance( what the fabric is does).

The Objectives of Textile Finishing are given below:

- To improve the attractiveness and service ability of the textile materials.

- To accentuate/inhibit some natural characteristics on the textile materials such as softening, de-lusturing, brightening etc.

- To change the surface characteristics of textile materials.

- To impart new characteristics/properties of textile materials such as flame retardant, water repellent or water proof finishes.

- To increase life and durability of textile materials.

- To set the fabric, so that it can be maintained its shape and structure.

- To set the chemicals into the textile materials.

- To meet up specific end uses.

Importance of Textile Finishes

Surface finishes not only make products look presentable and unique, but they also play an important role in ensuring products perform as they are intended to perform and last longer. Finishes provide protection from outside elements, corrosion, wear and rust.

Textile finishing decides the ultimate appearance and aesthetic properties of textile material. Additionally, it is liable to change various physical and chemical properties of textile materials per consumer needs. Textiles are made with a wide variety of fibre compositions, yarns and fabric structures.

•improve the appearance of fabric and enhance its looks;

•produce variety in fabrics through dyeing and printing;

•improve the feel or touch of fabric;

•make the fabric more useful;

•improve the draping ability of light weight fabrics;

•Make fabric suitable for an end (specific) use.

## Functional Finish

This type of finish changes the internal performance of the fabric. These are given to improve the aesthetic purpose of a fabric. Thus functional finishes can be further divided into two groups :

1.           Aesthetic Finish

It is given to enhance the fabric appearance and its draping ability. Some of the important aesthetic finishes are :

a.       Napping and scudding

b.       Mercerization

c.       Shearing

d.       Softening

e.       Stiffening

2.           PERFORMANCE FINISH

It is given to enrich the fabric properties like strength and durability. Some of the important functional finishes are :

a.       Antimicrobial

b.       Moth proof

c.       Crease resistant

d.       Durable press

e.       Soil resistant

f.         Water repellent or water proof

g.       Shrink proofing

h.       Flame resistant

## Quality Finish

hese types of finishes improve the quality of the fabric based on the time line or usage. They can be temporary, semi-permanent or permanent.

1. Temporary Finish

As the name suggest the temporary finish is not a stable finish and it disappears immediately after the first wash or first few washes. It includes finishes like :

               Starching or sizing

               Softening

               Embossing

               Calendaring

2. Semi-Permanent Finish

This finishing is durable than temporary finish and can withstand more than 10 to 15 washes. It includes finishes like:

               Schreiner calendaring

               Buckram finish

3. Permanent Finish

Permanent finishing does not disappear and remains unaffected through all the conditions of wear and washing treatments given to the fabric. It include finishes like:

               Flame retardant

               Resin finish

               Sanforising

               Water proof or water repellent

CHEMICAL AND MECHANICAL FINISHES / WET AND DRY FINISHES

On the basis of processes involved in application of finish,

there are two types – chemical (wet) and mechanical (dry) finishes.

I . MECHANICAL FINISHES:

These are also known as dry finishes. Here the process consists of application of moisture, pressure and heat or a mechanical device to finish a fabric. Beating, brushing, calendaring, filling, etc. are some of the finishes included in this group. These finishes are either temporary or semi durable and do not last long.

Calendering
A process of passing cloth between rollers (or “calendars”), usually under carefully controlled heat and pressure, to produce a variety of surface textures or effects in fabric such as compact, smooth, supple, flat and glazed.

Compacting
Durable finish imparted on man-made fibers and knitted fabrics by employing heat and pressure to shrink them to produce a crêpey and bulky texture.

Embossing
This particular type of calendering process allows engraving a simple pattern on the fabric. To produce a pattern in relief by passing fabric through a calendar in which a heated metal bowl engraved with the pattern works against a relatively soft bowl, built up of compressed paper or cotton on a metal centre.

Sueding
Sueding finishing process is carried out by means of a roller coated with abrasive material. The fabric has a much softer hand and an improved insulating effect thanks to the fiber end pulled out of the fabric surface.

Raising or Napping
The raising of the fiber on the face of the goods by means of teasels or rollers covered with card clothing (steel wires) that are about one inch in height. Action by either method raises the protruding fibers and causes the finished fabric to provide greater warmth to the wearer makes the cloth more compact, causes the fabric to become softer in hand or smoother in feel; increase durability and covers the minute areas between the interlacing of the warp and the filling. Napped fabrics include blankets, flannel, unfinished worsted, and several types of coatings and some dress goods. Other names for napping are Gigging, Genapping, Teaseled, Raised.

Shearing
Shearing is an important preparatory stage in the processing of cotton cloth. The objective of “Shearing” is to remove fibers and loose threads from the surface of the fabric, thus improving surface finish.

Stabilization
A term usually referring to fabrics in which the dimensions have been set by a suitable preshrinking operation

II CHEMICAL FINISHES:

These are also known as wet finishes. In these, chemical treatment is given to fabric, either to change its appearance or basic properties. It is the application or deposition of chemical compounds into the fibers to improve the attractiveness and or serviceability. Sometimes fibers react with the chemical compounds l and form covalent bonds and the finishing effect can withstand several laundering operations. This type of finishing is often known as durable or permanent finishing. Examples are: fire proof, crease resistance, etc.

Elastomeric finishes
Elastomeric finishes are also referred to as stretch or elastic finishes and are particularly important for knitwear. These finishes are currently achieved only with silicone-based products.

Crease resistant or Crease proofing
Crease Resistant Finishes are applied to cellulose fibers (cotton, linen and rayon) that wrinkle easily. Permanent Press fabrics have crease resistant finishes that resist wrinkling and also help to maintain creases and pleats throughout wearing and cleaning.

Soil release finishes
These finishes attract water to the surface of fibers during cleaning and help remove soil.

Flame retardant treatment
Are applied to combustible fabrics used in children’s sleepwear, carpets and curtains and prevent highly flammable textiles from bursting into flame.

Peach finish
Subjecting the fabric (either cotton or its synthetic blends) to emery wheels, makes the surface velvet like. This is a special finish mostly used in garments.

Anti Pilling
Pilling is a phenomenon exhibited by fabrics formed from spun yarns (yarns made from staple fibers). Pills are masses of tangled fibers that appear on fabric surfaces during wear or laundering. Fabrics with pills have an unsightly appearance and an unpleasant handle. Loose fibers are pulled from yarns and are formed into spherical balls by the frictional forces of abrasion. These balls of tangled fibers are held to the fabric surface by longer fibers called anchor fibers. Anti pilling finish reduces the forming of pills on fabrics and knitted products made from yarns with synthetic-fiber content.

Non slip finish
A finish applied to a yarn to make it resistant to slipping and sliding when in contact with another yarn.

FINISHING GIVES THE FOLLOWING ADVANTAGES:
1. Improved appearance – Lustre, Whiteness etc.,
2. Improved Feel which depends on the handle of the fabric and its Softness, Suppleness, Fullness etc.,
3. It improves the wearing qualities – Non soiling, Anticrease.
4. It gives special properties required for particular uses - Water proofing, Flame proofing etc.,
5. It covers the faults of the original cloth.
6. It increases the weight of the fabric.
7. It increases the sale value of the fabric.

DISADVANTAGES:

Relatively high cost

Possibility to reduce mechanical properties

Yellowing effect

Some finishes are not environmentally friendly

Toxic chemical usage

Corrosion of equipment

Unpleasant odor in working environment.

High capital equipment cost.

Greater care and skill is necessary.

Limited range of applicable product.

In some cases problem with penetration and levelness.